Instead of adjusting the setting on all the phones of the remote users. Create a line group just for remote users. See the instructions below.

Any questions, comments, or concerns, please reach out. Think of this as a working document.

- Create a SIP line or lines to match your PSTN circuits. Make sure they have remote users in the name.

- Make the line outbound only

- For the outbound identity, use the companies preferred number

- Change the following values to “Use line value 1”

- Assign the outbound proxy IP. This can be an SBC or gateway.

- For access deny all inbound traffic

- If the customer is using location assign location.

- Create a line group for the remote users.

- Use as a dial group only

- Add the SIP line you want to use for remote users.

- Assign the line group to be used by remote users.